Refactoring for Performance Without Approximations
A Developer Guide and Case Study

Example: ~30% speedup via low-level optimization (pipeline fusion + hot-loop refactor)

# Overview

This document describes practical refactoring techniques for improving performance while preserving the original algorithm and numerical intent. The case study shows a ~30% speedup achieved by reducing memory traffic, eliminating redundant passes over image data, restructuring hot loops to be more compiler- and SIMD-friendly, and precomputing reusable geometry and addressing.

## Scope and non-goals

Goal: speed up the existing implementation without changing the algorithm (no approximations or model changes).

Goal: preserve behavior up to normal floating-point roundoff effects.

Non-goal: introduce new math shortcuts, heuristic thresholds (beyond existing guards), or alternate CTF models.

Non-goal: redesign the pipeline at the architecture level (this is a low-level refactor).

# Case Study 1: Fusing the preprocessing pipeline (prepimg4align)

The original preprocessing pipeline performs multiple full-image passes: normalization, FFT, clipping, spatial shift, IFFT, masking, optional division by an instrument function, and a final FFT. The refactor preserves the same pipeline semantics but consolidates most full-array work into two fused kernels.

## Before: prepimg4align (baseline)

!>  \brief  prepares one particle image for alignment
!!          serial routine
subroutine prepimg4align( iptcl, img, img_out )
    use simple_ctf, only: ctf
    integer,      intent(in)    :: iptcl
    class(image), intent(inout) :: img
    class(image), intent(inout) :: img_out
    type(ctf)       :: tfun
    type(ctfparams) :: ctfparms
    real            :: x, y, sdev_noise, crop_factor
    ! Normalise
    if( params_glob%l_noise_norm )then
        call img%norm_noise(build_glob%lmsk, sdev_noise)
    else
        call img%norm_within(build_glob%lmsk)
    endif
    ! Fourier cropping
    call img%fft()
    call img%clip(img_out)
    ! Shift image to rotational origin
    crop_factor = real(params_glob%box_crop) / real(params_glob%box)
    x = build_glob%spproj_field%get(iptcl, 'x') * crop_factor
    y = build_glob%spproj_field%get(iptcl, 'y') * crop_factor
    if(abs(x) > SHTHRESH .or. abs(y) > SHTHRESH)then
        call img_out%shift2Dserial([-x,-y])
    endif
    ! Phase-flipping
    ctfparms = build_glob%spproj%get_ctfparams(params_glob%oritype, iptcl)
    select case(ctfparms%ctfflag)
        case(CTFFLAG_NO, CTFFLAG_FLIP)
            ! nothing to do
        case(CTFFLAG_YES)
            ctfparms%smpd = ctfparms%smpd / crop_factor != smpd_crop
            tfun          = ctf(ctfparms%smpd, ctfparms%kv, ctfparms%cs, ctfparms%fraca)
            call tfun%apply_serial(img_out, 'flip', ctfparms)
        case DEFAULT
            THROW_HARD('unsupported CTF flag: '//int2str(ctfparms%ctfflag)//' prepimg4align')
    end select
    ! Back to real space
    call img_out%ifft
    ! Soft-edged mask
    if( params_glob%l_focusmsk )then
        if( params_glob%l_needs_sigma )then
            call img_out%mask(params_glob%focusmsk*crop_factor, 'softavg')
        else
            call img_out%mask(params_glob%focusmsk*crop_factor, 'soft')
        endif
    else
        if( params_glob%l_needs_sigma )then
            call img_out%mask(params_glob%msk_crop, 'softavg')
        else
            call img_out%mask(params_glob%msk_crop, 'soft')
        endif
    endif
    ! gridding prep
    if( params_glob%gridding.eq.'yes' ) call build_glob%img_crop_polarizer%div_by_instrfun(img_out)
    ! return to Fourier space
    call img_out%fft()
end subroutine prepimg4align

## After: prepimg4align (refactored)

!>  \brief  prepares one particle image for alignment
!          serial routine
subroutine prepimg4align( iptcl, img, img_instr, img_out )
    use simple_ctf, only: ctf
    integer,      intent(in)    :: iptcl
    class(image), intent(inout) :: img
    class(image), intent(in)    :: img_instr
    class(image), intent(inout) :: img_out
    type(ctf)       :: tfun
    type(ctfparams) :: ctfparms
    real            :: shvec(2), crop_factor
    crop_factor = real(params_glob%box_crop) / real(params_glob%box)
    shvec(1)    = -build_glob%spproj_field%get(iptcl, 'x') * crop_factor
    shvec(2)    = -build_glob%spproj_field%get(iptcl, 'y') * crop_factor
    ! fused noise normalization, FFT, clip & shift
    call img%norm_noise_fft_clip_shift(build_glob%lmsk, img_out, shvec)
    ! Phase-flipping
    ctfparms = build_glob%spproj%get_ctfparams(params_glob%oritype, iptcl)
    select case(ctfparms%ctfflag)
        case(CTFFLAG_NO, CTFFLAG_FLIP)
            ! nothing to do
        case(CTFFLAG_YES)
            ctfparms%smpd = ctfparms%smpd / crop_factor != smpd_crop
            tfun          = ctf(ctfparms%smpd, ctfparms%kv, ctfparms%cs, ctfparms%fraca)
            call tfun%apply_serial(img_out, 'flip', ctfparms)
        case DEFAULT
            THROW_HARD('unsupported CTF flag: '//int2str(ctfparms%ctfflag)//' prepimg4align')
    end select
    ! fused IFFT, masking, division with instrument function & FFT
    call img_out%ifft_mask_divwinstrfun_fft(params_glob%msk_crop, img_instr)
end subroutine prepimg4align

## Refactoring principles applied

Fuse adjacent stages that each stream over the full image: normalization, fftshift, FFT, clip, and shift were combined into one kernel; IFFT, mask, instrument-function division, and FFT were combined into another.

Replace an explicit spatial shift with its mathematically equivalent Fourier-domain phase modulation applied during clipping (exact identity; avoids an extra resampling pass).

Piggyback data rearrangements (fftshift) onto mandatory passes (e.g., apply normalization during the quadrant swap) to reduce the number of pure permutation passes.

Short-circuit no-op or near-no-op cases with cheap guards (e.g., skip expensive normalization arithmetic when mean ~ 0 and invstd ~ 1; skip shift when below threshold).

Make memory access patterns explicit and predictable by operating directly on the underlying arrays inside the fused kernels.

Keep high-level readability at the call site: the refactored prep function remains a short, readable description of the pipeline.

## Fused kernels: design notes

The fused kernels are intentionally narrow and specialized: they capture a common pipeline fragment that was previously expressed as multiple method calls. This reduces memory traffic and call overhead while keeping the public API stable and the high-level pipeline readable.

Normalization performed as a two-pass reduction (double precision accumulation), followed by single-precision application.

fftshift combined with normalization by swapping quadrants and applying the affine transform on-the-fly.

Fourier shift implemented with a phase recurrence to avoid per-element trig calls.

Masking uses a computed background average (softavg) and fuses the mask with division by the instrument function in a single streaming pass.

# Case Study 2: Hot-loop refactor for CTF application (apply_serial)

The original CTF application used a mode-dependent select-case that duplicated nearly identical nested loops. The refactor preserves the same modes but restructures computation to improve reuse, reduce branching in the hot loop, and enable compiler vectorization.

## Before: apply_serial (baseline)

!>  \brief  is for optimised serial application of CTF
!!          modes: abs, ctf, flip, flipneg, neg, square
subroutine apply_serial( self, img, mode, ctfparms )
    use simple_image, only: image
    class(ctf),       intent(inout) :: self        !< instance
    class(image),     intent(inout) :: img         !< image (output)
    character(len=*), intent(in)    :: mode        !< abs, ctf, flip, flipneg, neg, square
    type(ctfparams),  intent(in)    :: ctfparms    !< CTF parameters
    integer :: lims(3,2),h,k,phys(3),ldim(3)
    real    :: ang,tval,spaFreqSq,hinv,kinv,inv_ldim(3)
    ! init object
    call self%init(ctfparms%dfx, ctfparms%dfy, ctfparms%angast)
    ! initialize
    lims     = img%loop_lims(2)
    ldim     = img%get_ldim()
    inv_ldim = 1./real(ldim)
    select case(mode)
        case('abs')
            do h=lims(1,1),lims(1,2)
                do k=lims(2,1),lims(2,2)
                    hinv      = real(h) * inv_ldim(1)
                    kinv      = real(k) * inv_ldim(2)
                    spaFreqSq = hinv * hinv + kinv * kinv
                    ang       = atan2(real(k),real(h))
                    tval      = self%eval(spaFreqSq, ang, 0.)
                    phys      = img%comp_addr_phys([h,k,0])
                    call img%mul_cmat_at(phys(1),phys(2),phys(3), abs(tval))
                end do
            end do
        ! ... other modes with similar loops ...
        case DEFAULT
            write(logfhandle,*) 'mode:', mode
            THROW_HARD('unsupported mode in apply_serial')
    end select
end subroutine apply_serial

## After: apply_serial (refactored)

! local fast CTF kernel (numerically equivalent refactor)
pure elemental real function fast_ctf_kernel( h, k, dfx, dfy, angast, amp_contr_const, wl, half_wl2_cs ) result(tval)
    integer, intent(in) :: h, k
    real,    intent(in) :: dfx, dfy, angast, amp_contr_const, wl, half_wl2_cs
    real :: df, evalPhSh, sum_df, diff_df, cterm, pi_wl_s2
    ! Defocus term: exactly the same expression, just factored
    sum_df  = dfx + dfy
    diff_df = dfx - dfy
    cterm   = cos( 2.0 * (mem_ctf_apply_mat(h,k)%ang - angast) )
    df      = 0.5 * ( sum_df + cterm * diff_df )
    ! Phase shift term: preserve the same evaluation structure
    pi_wl_s2 = PI * wl * mem_ctf_apply_mat(h,k)%spaFreqSq
    evalPhSh = pi_wl_s2 * (df - half_wl2_cs * mem_ctf_apply_mat(h,k)%spaFreqSq)
    tval     = sin( evalPhSh + amp_contr_const )
end function fast_ctf_kernel

!>  \brief  is for optimised serial application of CTF
!!          modes: abs, ctf, flip, flipneg, neg, square
subroutine apply_serial( self, img, mode, ctfparms )
    class(ctf),       intent(inout) :: self     !< instance
    class(image),     intent(inout) :: img      !< image (output)
    character(len=*), intent(in)    :: mode     !< abs, ctf, flip, flipneg, neg, square
    type(ctfparams),  intent(in)    :: ctfparms !< CTF parameters
    real, parameter :: ZERO = 0., ONE = 1.0, MIN_SQUARE = 0.001
    integer :: h,k
    real    :: dfx, dfy, angast, amp_contr_const, wl, half_wl2_cs, tval, t
    logical :: is_abs,is_ctf,is_flip,is_flipneg,is_neg,is_square
    ! Convert mode string to logical flags (SIMD-compatible)
    is_abs     = (mode == 'abs')
    is_ctf     = (mode == 'ctf')
    is_flip    = (mode == 'flip')
    is_flipneg = (mode == 'flipneg')
    is_neg     = (mode == 'neg')
    is_square  = (mode == 'square')
    ! initialize
    call self%init(ctfparms%dfx, ctfparms%dfy, ctfparms%angast) ! conversions
    wl              = self%wl
    half_wl2_cs     = 0.5 * wl * wl * self%cs
    dfx             = self%dfx
    dfy             = self%dfy
    angast          = self%angast
    amp_contr_const = self%amp_contr_const
    do h = lims_memo(1,1), lims_memo(1,2)
        do k = lims_memo(2,1), lims_memo(2,2)
            ! calculate CTF
            tval = fast_ctf_kernel(h, k, dfx, dfy, angast, amp_contr_const, wl, half_wl2_cs)
            ! SIMD-compatible mode handling using masked operations
            t = merge(       abs(tval),                     ZERO, is_abs)     + &
                merge(           tval,                      ZERO, is_ctf)     + &
                merge( sign(ONE, tval),                     ZERO, is_flip)    + &
                merge(-sign(ONE, tval),                     ZERO, is_flipneg) + &
                merge(          -tval,                      ZERO, is_neg)     + &
                merge(min(ONE, max(tval*tval, MIN_SQUARE)), ZERO, is_square)
            ! Multiply (this is the key operation)
            call img%mul_cmat_at(mem_ctf_apply_mat(h,k)%ph1,mem_ctf_apply_mat(h,k)%ph2,1, t)
        end do
    end do
end subroutine apply_serial

## Refactoring principles applied

Unify duplicated loop bodies: compute the CTF value once and apply the mode-specific transform via masked operations rather than separate loops.

Reduce branching in the hot loop: mode selection is converted into precomputed logical flags and merge-based masking.

Hoist loop-invariant constants (wavelength, defocus parameters, constants derived from cs) outside the loops.

Memoize per-frequency geometry and addressing (angle, spatial frequency squared, physical index mapping) to avoid repeated atan2/divides/address calculations.

Extract a pure elemental kernel function to make the core math explicit, testable, and compiler-friendly (inlining/vectorization).

# Ensuring numerical equivalence

These refactors preserve the algorithm. Differences should be limited to expected floating-point roundoff. Recommended validation steps:

Unit test fused kernels against the original pipeline for representative images and masks (cover shift/no-shift paths and different mask modes).

Compare outputs in both real and Fourier space; compute max-abs error and relative error norms.

Run end-to-end regression tests on a small, fixed dataset and confirm downstream alignment metrics are unchanged within tolerance.

Lock down FFTW plan configuration (precision, flags) to avoid confounding performance/precision changes.

# Performance checklist for similar refactors

Profile and quantify where time goes; target the few stages that dominate runtime.

Fuse consecutive full-array operations to minimize memory bandwidth pressure and cache churn.

Avoid repeated domain round-trips; keep data in the representation needed for the next step.

Replace per-element transcendentals with recurrence or memoization.

Hoist invariants; precompute index maps and geometry used in inner loops.

Prefer uniform, SIMD-friendly loops over mode-specific branching.

Keep readability at the orchestration level; hide low-level kernels behind well-named APIs.

Appendix: Notes on maintainability

Fused kernels should be narrow and well-commented. Treat them as performance-critical building blocks with strong tests. Keep the high-level pipeline readable (as in the refactored prepimg4align) so future changes remain safe.